#include <Servo.h>

Servo myservo[10];

double w = 9.0;

double a = 8.3;

double b = 7.7;

double c = 6.3;

double d = 4.5;

double e = 1.5;

double H = 28.3; // total height of the Robot

double S = 25.0; // siting height of the Robot

double h = 28.3; // current height of the Robot

double X0 = 15; // length of a full step

double Y0 = 0.54*w; // amount of left/right shift of hip during walking

double Z0 = 3.0; // full leg up length during walking

double t0 = 1.0; // full time to complete single  action

double PhipCenter[3] = {0, 0, 0};

double PfootL[3] = {0, 0, 0};

double PfootR[3] = {0, 0, 0};

double NormalMotorPosition[] = {87, 95, 80, 90, 88, 90, 90, 108, 90, 90};

double currentMotorPosition[10] = {87, 95, 80, 90, 88, 90, 90, 108, 90, 90};

double FinalMotorPosition[10] = {};

void setup() {

// put your setup code here, to run once:

Serial.begin(9600);

for(int i=0; i<10; i++){

myservo[i].attach(i+2);

}

setMotors(NormalMotorPosition);

StandAtHeight(25);

delay(1000);

}

void loop() {

// put your main code here, to run repeatedly:

StartByLeftLeg();

WalkByRightLeg();

WalkByLeftLeg();

WalkByRightLeg();

WalkByLeftLeg();

WalkByRightLeg();

WalkByLeftLeg();

StopByRightLeg();

delay(100000);

}

void StandAtHeight(double height){

double t=0.0;

while(t<=t0/4){

StandingAtHeight(t, height, PhipCenter, PfootL, PfootR);

CalculateMotorPosistion(PhipCenter, PfootL, PfootR, FinalMotorPosition);

setMotors(FinalMotorPosition);

t+=0.05;

}

h = height;

}

void StartByLeftLeg(){

double t=0.0;

while(t<=t0){

LeftLegStarting(t, PhipCenter, PfootL, PfootR);

CalculateMotorPosistion(PhipCenter, PfootL, PfootR, FinalMotorPosition);

setMotors(FinalMotorPosition);

t+=0.05;

}

}

void WalkByRightLeg(){

double t=0.0;

while(t<=t0){

RightLegWalking(t, PhipCenter, PfootL, PfootR);

CalculateMotorPosistion(PhipCenter, PfootL, PfootR, FinalMotorPosition);

setMotors(FinalMotorPosition);

t+=0.05;

}

}

void WalkByLeftLeg(){

double t=0.0;

while(t<=t0){

LeftLegWalking(t, PhipCenter, PfootL, PfootR);

Serial.println(String(PhipCenter[0]) + " " + String(PhipCenter[1]) + " " + String(PhipCenter[2]) );

Serial.println(String(PfootL[0]) + " " + String(PfootL[1]) + " " + String(PfootL[2]) );

Serial.println(String(PfootR[0]) + " " + String(PfootR[1]) + " " + String(PfootR[2]) );

Serial.println();

CalculateMotorPosistion(PhipCenter, PfootL, PfootR, FinalMotorPosition);

setMotors(FinalMotorPosition);

t+=0.05;

}

}

void StopByRightLeg(){

double t=0.0;

while(t<=t0+0.01){

RightLegStoping(t, PhipCenter, PfootL, PfootR);

CalculateMotorPosistion(PhipCenter, PfootL, PfootR, FinalMotorPosition);

setMotors(FinalMotorPosition);

t+=0.05;

}

}

///////////////////////////////////////////////////////////////////////////////////////////////////////

//////////////////////////////////  Generate trajectories  ////////////////////////////////////////////

void StandingAtHeight(double t, double height, double (&PhipCenter)[3], double (&PfootL)[3], double (&PfootR)[3]){

double z = h - (3.0*(h - height)*pow(t, 2))/pow(t0/4, 2) + (2.0*(h - height)*pow(t, 3))/pow(t0/4, 3);

PhipCenter[0] = 0.0;

PhipCenter[1] = 0.0;

PhipCenter[2] = 0.0;

PfootL[0] = PhipCenter[0];

PfootL[1] = PhipCenter[1]-w/2;

PfootL[2] = z;

PfootR[0] = PhipCenter[0];

PfootR[1] = PhipCenter[1]+w/2;

PfootR[2] = z;

}

void LeftLegStarting(double t, double (&PhipCenter)[3], double (&PfootL)[3], double (&PfootR)[3]){

double t1 = 0.3*t0;

double t2 = t0-t1;

if(t<t1){

PhipCenter[0] = 0.0;

PhipCenter[1] = (3.0*(Y0)*pow(t, 2))/pow(t1, 2) - (2.0*(Y0)*pow(t, 3))/pow(t1, 3);

} else if(t<t2){

PhipCenter[0] = 0.0;

PhipCenter[1] = Y0;

} else {

PhipCenter[0] = (3.0*(X0/4)*pow((t-t2), 2))/pow((t0-t2), 2) - (2.0*(X0/4)*pow((t-t2), 3))/pow((t0-t2), 3);

PhipCenter[1] = Y0-4*(Y0/X0)*PhipCenter[0];

}

PhipCenter[2] = 0.0;

t1 = 0.25*t0;

t2 = 0.35*t0;

double t3 = 0.7*t0;

if(t<t1){

PfootL[0] = 0.0;

PfootL[2] = h;

} else if(t<t2){

PfootL[0] = 0.0;

PfootL[2] = h - (3.0*(Z0)*pow((t-t1), 2))/pow((t2-t1), 2) + (2.0*(Z0)*pow((t-t1), 3))/pow((t2-t1), 3);

} else if(t<t3){

PfootL[0] = (3.0*(X0/2)*pow((t-t2), 2))/pow((t3-t2), 2) - (2.0*(X0/2)*pow((t-t2), 3))/pow((t3-t2), 3);

PfootL[2] = h - Z0*sqrt(abs(1 - pow((2*PfootL[0]/X0), 2)));

} else {

PfootL[0] = X0/2;

PfootL[2] = h;

}

PfootL[1] = -w/2;

PfootR[0] = 0.0;

PfootR[1] = w/2;

PfootR[2] = h;

}

void RightLegWalking(double t, double (&PhipCenter)[3], double (&PfootL)[3], double (&PfootR)[3]){

double t1 = 0.3*t0;

double t2 = t0-t1;

if(t<t1){

PhipCenter[0] = -X0/4 + (3.0*(X0/4)*pow((t), 2))/pow((t1), 2) - (2.0*(X0/4)*pow((t), 3))/pow((t1), 3);

PhipCenter[1] = -Y0-4*(Y0/X0)*PhipCenter[0];

} else if(t<t2){

PhipCenter[0] = 0.0;

PhipCenter[1] = -Y0;

} else {

PhipCenter[0] = (3.0*(X0/4)*pow((t-t2), 2))/pow((t0-t2), 2) - (2.0*(X0/4)*pow((t-t2), 3))/pow((t0-t2), 3);

PhipCenter[1] = -Y0+4*(Y0/X0)*PhipCenter[0];

}

PhipCenter[2] = 0.0;

t1 = 0.25*t0;

t2 = 0.75*t0;

if(t<t1){

PfootR[0] = -X0/2;

PfootR[2] = h;

} else if(t<t2){

PfootR[0] = -X0/2 + (3.0*(X0)*pow((t-t1), 2))/pow((t2-t1), 2) - (2.0*(X0)*pow((t-t1), 3))/pow((t2-t1), 3);

PfootR[2] = h - Z0*sqrt(abs(1 - pow((2*PfootR[0]/X0), 2)));

} else {

PfootR[0] = X0/2;

PfootR[2] = h;

}

PfootR[1] = w/2;

PfootL[0] = 0.0;

PfootL[1] = -w/2;

PfootL[2] = h;

}

void LeftLegWalking(double t, double (&PhipCenter)[3], double (&PfootL)[3], double (&PfootR)[3]){

double t1 = 0.3*t0;

double t2 = t0-t1;

if(t<t1){

PhipCenter[0] = -X0/4 + (3.0*(X0/4)*pow((t), 2))/pow((t1), 2) - (2.0*(X0/4)*pow((t), 3))/pow((t1), 3);

PhipCenter[1] = Y0+4*(Y0/X0)*PhipCenter[0];

} else if(t<t2){

PhipCenter[0] = 0.0;

PhipCenter[1] = Y0;

} else {

PhipCenter[0] = (3.0*(X0/4)*pow((t-t2), 2))/pow((t0-t2), 2) - (2.0*(X0/4)*pow((t-t2), 3))/pow((t0-t2), 3);

PhipCenter[1] = Y0-4*(Y0/X0)*PhipCenter[0];

}

PhipCenter[2] = 0.0;

t1 = 0.25*t0;

t2 = 0.75*t0;

if(t<t1){

PfootL[0] = -X0/2;

PfootL[2] = h;

} else if(t<t2){

PfootL[0] = -X0/2 + (3.0*(X0)*pow((t-t1), 2))/pow((t2-t1), 2) - (2.0*(X0)*pow((t-t1), 3))/pow((t2-t1), 3);

PfootL[2] = h - Z0*sqrt(abs(1 - pow((2*PfootL[0]/X0), 2)));

} else {

PfootL[0] = X0/2;

PfootL[2] = h;

}

PfootL[1] = -w/2;

PfootR[0] = 0.0;

PfootR[1] = w/2;

PfootR[2] = h;

}

void RightLegStoping(double t, double (&PhipCenter)[3], double (&PfootL)[3], double (&PfootR)[3]){

double t1 = 0.3*t0;

double t2 = t0-t1;

if(t<t1){

PhipCenter[0] = -X0/4 + (3.0*(X0/4)*pow((t), 2))/pow((t1), 2) - (2.0*(X0/4)*pow((t), 3))/pow((t1), 3);

PhipCenter[1] = -Y0-4*(Y0/X0)*PhipCenter[0];

} else if(t<t2){

PhipCenter[0] = 0.0;

PhipCenter[1] = -Y0;

} else {

PhipCenter[0] = 0.0;

PhipCenter[1] = -Y0 + (3.0*(Y0)*pow((t-t2), 2))/pow((t0-t2), 2) - (2.0*(Y0)*pow((t-t2), 3))/pow((t0-t2), 3);

}

PhipCenter[2] = 0.0;

t1 = 0.30*t0;

t2 = 0.65*t0;

double t3 = 0.75*t0;

if(t<t1){

PfootR[0] = -X0/2;

PfootR[2] = h;

} else if(t<t2){

PfootR[0] = -X0/2 + (3.0*(X0/2)*pow((t-t1), 2))/pow((t2-t1), 2) - (2.0*(X0/2)*pow((t-t1), 3))/pow((t2-t1), 3);

PfootR[2] = h - Z0*sqrt(abs(1 - pow((2*PfootR[0]/X0), 2)));

} else if(t<t3){

PfootR[0] = 0.0;

PfootR[2] = h - Z0 + (3.0*(Z0)*pow((t-t2), 2))/pow((t3-t2), 2) - (2.0*(Z0)*pow((t-t2), 3))/pow((t3-t2), 3);

} else {

PfootR[0] = 0.0;

PfootR[2] = h;

}

PfootR[1] = w/2;

PfootL[0] = 0.0;

PfootL[1] = -w/2;

PfootL[2] = h;

}

///////////////////////////////////////////////////////////////////////////////////////////////////////

//////////////////////////////////  Calculate motor position  /////////////////////////////////////////

void CalculateMotorPosistion(double PhipCenter[], double PfootL[], double PfootR[], double (&FinalMotorPosition)[10]){

double q[5] = {0, 0, 0, 0, 0};

double PhipL[3] = {PhipCenter[0], PhipCenter[1]-w/2, PhipCenter[2]};

InverseKinematics(PhipL, PfootL, q);

for(int i=0; i<5; i++){

FinalMotorPosition[i] = NormalMotorPosition[i] + q[i]*(180.0/3.14);

}

double PhipR[3] = {PhipCenter[0], PhipCenter[1]+w/2, PhipCenter[2]};

InverseKinematics(PhipR, PfootR, q);

for(int i=0; i<5; i++){

FinalMotorPosition[5+i] = NormalMotorPosition[5+i] - q[i]*(180.0/3.14);

}

FinalMotorPosition[5] = NormalMotorPosition[5] + q[0]*(180.0/3.14);

FinalMotorPosition[9] = NormalMotorPosition[9] + q[4]*(180.0/3.14);

}

void InverseKinematics(double Phip[], double Pfoot[], double (&q)[5]){

double x = Pfoot[0] - Phip[0];

double y = Pfoot[1] - Phip[1];

double z = Pfoot[2] - Phip[2];

q[0] = -atan(y/(z-e));

q[4] = q[0];

double a1 = a*cos(q[0]);

double b1 = b*cos(q[0]);

double c1 = c*cos(q[0]);

double d1 = d*cos(q[0]);

double z1 = z - a1 - d1 - e;

double r = sqrt(x*x + z1*z1);

double phi1 = atan(x/z1);

double phi2 = acos((b1*b1 + r*r - c1*c1)/(2*b1*r));

q[3] = phi1 + phi2;

double phi3 = acos((b1*b1 + c1*c1 - r*r)/(2*b1*c1));

q[2] = - phi3 + 3.14;

q[1] = -q[3] + q[2];

}

///////////////////////////////////////////////////////////////////////////////////////////////////////

//////////////////////////////////  Set motor position  ///////////////////////////////////////////////

void setMotors(double finalMotorPosition[]){

for(int t=0; t<=100; t++){

for(int i=0; i<10; i++){

int angle = currentMotorPosition[i] + t*(finalMotorPosition[i] - currentMotorPosition[i])/100;

myservo[i].write(angle);

}

delay(2);

}

for(int i=0; i<10; i++){

currentMotorPosition[i] = finalMotorPosition[i];

}

}